A Brief Documentation of the Driver Station and Associated Controls

General Notes

- Any reference to the LCD Screen below refers to the “User Messages” box under the Operation tab.

Driver Station – General

- Setting up Joysticks: If you go to the Setup tab, you should see “Logitech Extreme 3D” in the first two slots and “Logitech Dual Action” in the third slot (if the game controller is plugged in). The first slot is for the Drive joystick and the second for the Operator joystick. If the joysticks are not in their proper slots, drag and drop them properly. To verify everything is correct, use the Operator joystick to try scrolling the “User Messages” display under the Operation tab. If no scrolling occurs, swap the two “Logitech Extreme 3D” slots under the Setup tab.

- Verifying Code is Running Properly: The first indication that the robot ahs code is on the left side of the Operation tab. The light next to “Robot Code” should be green. However, sometimes code may stall or crash. To verify that code is actually running: 1)Scroll to the top of the LCD screen 2)The top left corner should have a spinning bar to verify that code is actually running.

Software Enhancements of Robot Subsystem Controls

- Arm and Lift Pot Protection: The lift and arm will not run if their respective pots return a value under 50 or above 950.

- Arm and Lift Physical Protection: The lift and arm have minimum and maximum pot values set in the config file. They will not move further out of bounds if the pot values are beyond the min/max values.

- Lift Stall Protection: If the lift is attempting to move (i.e. an input is being sent), but the pot does not see the lift moving for about half a second, the lift will be disabled to protect the motor from stalling. To re-enable the lift, let go of the button if you were manually powering it, or quickly tap either of the manual buttons if the lift was running to a preset.

- Lift Pulsing near Physical Limits: If the lift is near its physical limits and moving towards the limit, it will pulse quickly to prevent physical damage. This pulsing behavior is expected.

- Arm Slower when Lift is Extended: If the lift is near fully extended and the arm is past its vertical position, the arm will move at a slower speed for greater control.

- Aborting a Preset Operation: There is an abort button on the Driver Stick (Button 5) that cancels all lift and arm operations. Alternately, tapping a button that manually controls the lift or arm will cancel its preset operation.

Robot Configuration

- In Brief: If you have changed a value in the .txt file via FTP or changed a value on one of the analog sliders under the I/O tab, do the following to reload the new config values: 1) Ensure the robot is disabled. 2)While holding down the trigger for the operator stick (Button 1), press Button 9 (“LOAD/ArmUp”).

- Using the Analog Sliders: If you want to use one of the four analog sliders (be careful, the 4th one is used for setting an autonomous delay), go into the .txt file via the FTP. Put a config identifier (e.g. “LRTesc.pgain”) as the value of one of the assignable sliders (numbered 0 thru 3). Be sure to set the range of the analog slider, as it will be rescaled for you from the interval [0,5] to an interval of your choice. Remember to ReloadConfig (Trigger + Button 9 on Operator Stick) to save your changes.

- Details: There are two places that config values are saved – the .txt file on the cRIO file system, and in memory in an STL map in the LRTConfig object. This distinction is essential to understanding the functionality of the three config buttons:

  - ApplyConfig (Trigger + Button 7 on Operator Stick): This makes all the subsystems reload their config values from the STL map. This is generally only used when updating a value set through an assignable slider.

  - ReloadConfig (Trigger + Button 9 on the Operator Stick): The LRTConfig object reloads all config values from the .txt file and then calls ApplyConfig so that all subsystems have their config values refreshed. This is used after changing something in the .txt file, and is the most often used button.

  - SaveConfig (Trigger + Button 11 on the Operator Stick): The LRTConfig object dumps all of its saved values into the .txt file. This is generally only used when a config value set through an assignable slider is satisfactory and needs to be saved back to the .txt file.

Autonomous

- Choosing an Autonomous Routine: The Digital Inputs under the I/O tab are used to select autonomous routines. As of March 16, 2010:

  - Digital In 3: Near Field Straight Kick

  - Digital In 8: Test Autonomous Routine

- Setting a Delay Before Autonomous Starts: This delay may only be set in real autonomous; no delay will be set for simulated autonomous in teleoperated mode (see below for details). Go to the I/O tab, and set the 4th analog slider to any value between 0 and 5. This corresponds to a delay of 0 to 5 seconds. Be sure that this slider is set to 0 if you do not want a delay!

- Simulating Autonomous in Teleoperated Mode: Autonomous Routines may be simulated in teleoperated mode for ease of testing. The game controller must be plugged in and enabled for this functionality (see below for details on the game controller). Use the Digital Inputs to select your autonomous routine (see above for details). Press Button 9 on the game controller to start the autonomous mode. The joysticks will not respond during this time. Press Button 10 on the game controller at any time to cancel the autonomous routine and regain joystick functionality. Note: You must press Button 10 after the autonomous routine, even if it has completed successfully, in order to regain joystick functionality.

Game Controller

- Plugging in the Game Controller: Plug in the game controller to any USB port. Go to the Setup tab in the Driver Station and ensure that “Logitech Dual Action” appears in the 3rd slot. Drag and drop as necessary.

- Enabling/Disabling the Game Controller: To ensure that the game controller does not conflict with the joysticks, only one will send signals to the robot at any time. To enable the game controller, press down on the left analog stick (Button 11), labeled “GC ON.” To disable the game controller and regain joystick functionality, press down on the right analog stick (Button 12), labeled “GC OFF.” Note: The game controller is automatically disabled if the robot is disabled, but is not automatically disabled if the game controller is unplugged.

- Determining Whether the Game Controller is Enabled: Scroll to the top of the LCD screen. If the game controller is enabled, you will see a “GC” printed on the top line of the LCD screen.

Different Modes in Teleoperated

- Visual Indicator: Scroll to the top of the LCD screen. You will see the mode we are in.

- Normal Mode: (Both throttles are down). This is the normal mode for operation. We should be in this mode most of the time.

- Test Mode: (Either throttle is up). Test mode is used only for testing the preset code for the arm. In test mode, the arm runs at a reduced speed when going to a preset, rather than full speed.

- Service Mode: (Both throttles are up). BE CAREFUL WITH THIS MODE! Pressing buttons randomly may cause presets to be overwritten. See the driver controls image for details on Service Mode controls

Downloading Software to Robot

- If you are getting errors while downloading software, try some of these fixes (they may or may not work, depending on the nature of your error):

  - Ensure that you can ping the robot.

  - If you are using the green button, ensure that no code has been flashed to the cRIO. Verify that there is no FRC_UserProgram.out running. If so, undeploy and reset the cRIO.

  - Try resetting the cRIO through the workbench: Right click -> Reset target server

  - Ensure that you have the latest Workbench Update installed

  - Give the cRIO a power cycle (may take a while for the cRIO to come back online if the game adapter takes a while to boot)

  - Worst case: Reimage the cRIO.